|  | Work at Height Permit | General Safe Work Permit #: ________________ Date: ______ / ______/______ |
| --- | --- | --- |

WORK AT HEIGHT PERMIT is required for heights of 4 ft or more when guardrails are not present.

| Section 1 – Type of Work at Height |  |
| --- | --- |
| ☐ | Climb up/down a fixed ladder of 24 ft or more (PERMIT IS NOT REQUIRED IF A FIXED LADDER IS LESS THAN 24 FT OR FOR PORTABLE LADDERS) |
| ☐ | Manlifts – any type of vehicle-mounted with a work platform. |
| ☐ | ALL other work locations above 4ft with no fixed fall protection system (e.g., with no guardrails) SPECIFY HERE: |

| Section 2 – Determination of Inspection Requirement |  |
| --- | --- |
| ☐ | **Go to Section 3** IF the work is conducted by a contractor / third party employees. |
| ☐ | **Go to Section 5** IF the work is conducted by a SACHEM employee with a Harness and a Lanyard that was INSPECTED and APPROVED TODAY, AND the BIANNUAL (i.e., every 6 months) inspection is up to date. |

| Section 3 – Harness & Lanyard of a Non-SACHEM employee |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Description of the pre-work inspection | PASS | FAIL | N/A |
|  | Full Body Harnesses & Lanyards |  |  |  |
|  | Harness AND Lanyard manufacturer’s identification tags are readable. |  |  |  |
|  | Harness AND Lanyard are NOT EXPIRED (Max. of 5 years, unless specified differently by the harness/lanyard manufacturer – harness/lanyard tags must contain the information.) |  |  |  |
|  | Harness AND Lanyard are complete in all respects i.e., no cut-offs or modifications. |  |  |  |
|  | Harness AND Lanyard have the ANSI Z359 certification tags. |  |  |  |
|  | Harness AND Lanyard are not soiled/damaged by chemicals, paints, etc. |  |  |  |
|  | Harness AND Lanyard have no fissures, wear, twisted straps, or seams open. |  |  |  |
|  | Harness AND Lanyard have no burn marks or heat exposure signs. |  |  |  |
|  | Shock / Energy Absorber Packet |  |  |  |
|  | No Fissures in the protective Sleeve. |  |  |  |
|  | Seams between shocks absorbers, lanyards, and hooks are intact. No signs of torn stitches. |  |  |  |
|  | Composite or Metal Fittings |  |  |  |
|  | Waist Buckle with No Deformation, Damage, or Corrosion. |  |  |  |
|  | Both Leg strap buckles with No Deformation, Damage, or Corrosion. |  |  |  |
|  | Back (the one at center of the shoulder blades) “D” ring with No Deformation, Damage, or Corrosion. |  |  |  |
|  | All other fittings are in good condition with No Deformation, Damage, or Corrosion. |  |  |  |
|  | ALL Snap Hooks & Carabiners |  |  |  |
|  | Snap hooks and Carabiners have the ANSI Z359 certification stamped on them. |  |  |  |
|  | Snap hooks and Carabiners have no fissures, deformation, or corrosion. |  |  |  |
|  | Snap hooks and Carabiners can only be opened by pushing the safety lever- i.e., they only open in a TWO-STAGE operation. They will automatically snap back (i.e., properly close) when released. |  |  |  |
| Section 4 – Ladder Inspection |  |  |  |  |
| 4.1. | Ladder inspection is up to date and rungs are free of grease, mud, or other slippery agents and have no visible defects or sign of damage. |  |  |  |
| 4.2. | A visible and recognizable barricade has been put in place at least 6 ft. around the work area to protect pedestrians from falling objects. |  |  |  |
| 4.3. | Wind gusts throughout the duration of the work at height are not expected to exceed 20 miles per hour. |  |  |  |

| Section 5 – Pre-Work Inspection | Section 6 – AUTHORIZATION - SACHEM Personnel ONLY |
| --- | --- |
| Personnel carrying out the work under this permit | Shift Supervisor (or Equivalent Representative) |
| - I certify that I WILL ONLY USE APPROVED AND INSPECTED HARNESS AND LANYARDS. - I certify that I INSPECTED MY LADDERS, HARNESS, LANYARD, AND OTHER APPLICABLE PERSONAL PROTECTIVE EQUIPMENT before this work started, and they are adequate and working properly according to their manufacturer’s design and recommendations. PRINT _________________________________________________ PRINT _________________________________________________ PRINT _________________________________________________ PRINT ________________________________________________ PRINT ________________________________________________ | I CERTIFY THAT ALL SAFETY CONCERNS WERE PROPERLY ADDRESSED, AND THE WORK CAN NOW PROCEED. NOTES: PRINT / Sign: ____________________________________ |

|  | Section 7 – PERMIT CLOSEOUT - SACHEM Personnel ONLY |
| --- | --- |
|  | Shift Supervisor (or Equivalent Representative) |
|  | I CONFIRM THAT ALL LADDERS, MECHANICAL EQUIPMENT, BARRICADES, AND OTHER ITEMS ASSOCIATED WITH THE WORK AT HEIGHT HAS BEEN REMOVED AFTER THE JOB COMPLETION. NOTES: PRINT / Sign: ____________________________________ |