| In case of emergency call Control room: |  | First break Permit |  |
| --- | --- | --- | --- |

General Safe Work Permit # : __________________________               Date:_______/ _______/_______

| Section 1 – Checklist must be dated and signed before continuing to first break. |  |  |  | Initials |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Yes | N/A | Prod. | Maint. |
| BEFORE the first break |  |  |  |  |  |
| 01 | The line receiving the first break last contained _________________________, This is a Hazardous &/or Flammable Non-Hazardous & Non-Flammable material. | X |  |  |  |
| 02 | SDS has been reviewed, by the associate(s) making the first break. |  |  |  |  |
| 03 | LOTO permit completed. The energy source for all applicable equipment has been locked out, tagged, and tried to ensure the power source has been de-energized. |  |  |  |  |
| 04 | If cutting a line – the line MUST be properly identified (ribbon/tags/spray paint) by the Project Manager AND Shift Supervisor AND Contractor. Identification must be no more than 3ft away from where the cutting activity will happen. Project Manager Initials __________________ Contractor Initials __________________ Shift Supervisors Initials __________________ If pipes are not identified do NOT proceed! Do NOT break the line. |  |  |  |  |
| 05 | Equipment/lines have been drained and vented (No pressure) |  |  |  |  |
| 06 | Equipment/lines have been cooled off (Equip./line with temperate ≤ 100 F) |  |  |  |  |
| 07 | Lines have been completely flushed and cleaned (No chemical liquids, vapors, or gases) |  |  |  |  |
| 08 | All associated valves/pumps have been closed, locked, and tagged (Double-check by walking the line if you were not the ones who completed the LOTO permit) |  |  |  |  |
| 09 | Production and Maintenance associates have discussed the steps needed to perform the task, discussed the hazards associated with the task, and determined a timeline as to when the first break can be performed. |  |  |  |  |
| 10 | If a barricade is required per the EHS Manual 22.7, it has been set up properly |  |  |  |  |
| 11 | Everyone involved in the work was communicated about the hazard of the chemical |  |  |  |  |
| 12 | The exact spot for the first break is agreed upon by the mechanic and operator. |  |  |  |  |
| 13 | Associate(s) making the first break has put on the proper PPE i.e., poly-coated Tyvek suit (or similar), gloves, goggles, and face shield. Review the SDS for respiratory protection. |  |  |  |  |
| 14 | With an operator close by, blinds have been installed so that all potential sources of chemical exposure have a solid, un-removable barrier as required. |  |  |  |  |
| AFTER the first break |  |  |  |  |  |
| 15 | Blinds are removed and the flanges bolted back up with new gaskets (if applicable). |  |  |  |  |
| 16 | Drain plugs have been placed back on the drain lines |  |  |  |  |
| 17 | The lines/connections have been leak-tested to ensure that there are no leaks |  |  |  |  |
| 18 | If leaks have been found, they were properly repaired |  |  |  |  |
| 19 | All locks and tags have been removed and the line or equipment has been turned back over to Production. |  |  |  |  |

| Section 2 – AUTHORIZATION to proceed with the FIRST BREAK | Section 3 – CLOSEOUT of the FRIST BREAK permit |
| --- | --- |
| Date & Time: ______ /______ /______ & _____:_____ | Date & Time: ______ /______ /______ & _____:_____ |
| Work has been AUTHORIZED by ____________________ (Prod Assoc.) and Turned over to ______________________ (Main Assoc.) | Work has been COMPLETED by _____________________ (Main Assoc.) and Turned over to ______________________ (Prod Assoc.) |